- Family Tree

- Blastware III

- Revision History

| Revision | Eco number | Pages Affected | Effective Date | Incorporated by |
| --- | --- | --- | --- | --- |
| 3.51b | 98018 | ALL | Jan. 28, 1998 | T.C |
| 3.60 | 98050 | ALL | Mar 18, 1998 | T.C |
| 3.61 | 98089 | ALL | Apr. 20, 1998 | T.C |
| 3.62 | 98113 | ALL | May 29, 1998 | T.C |
| 3.74 | 98266 | ALL | Nov.10, 1998 | KW |
| 4.0 | 99163 | ALL | May 14, 1999 | TC |
| 4.02 | 99195 | ALL | June 15, 1999 | SS |
| 4.30 | 01001 | All | Jan 19, 2001 | SS |
| 4.33 | 01137 | ALL | July 12, 2001 | SS |
| 4.37 | 03104 | ALL | Mar 24, 2003 | SS |
| 7.0 | 03200 | ALL | Sept 5, 2003 | SS |
| 7.01 | 03216 | ALL | Sept 26, 2003 | SS |
| 7.10 | 30788 | ALL | Dec 17, 2003 | SS |
| 8.00 | 31276 | ALL | Nov 19, 2004 | SS |
| 8.01 | 31292 | ALL | Dec 3, 2004 | SS |
| 8.02 | 31341 | ALL | Jan 11, 2005 | SS |
| 8.03 |  | ALL | Feb 10, 2005 | SS |
| 804 | 31949 (BETA) | ALL | Nov 2, 2005 | SS |
| 8.10 | 32712 | ALL | Jan 12, 2007 | SS |
| 8.11 | 32793 | ALL | Feb 9, 2007 | SS |
| 8.12 | 32814 | ALL | Feb 20, 2007 | SS |
| 10.01 | 34511 | ALL | March 4 2009 |  |
| 10.02 | 34680 | ALL | April 3, 2009 |  |
| 10.03 |  | ALL | May 8, 2009 |  |
| 10.04 |  | ALL | June 24, 2009 |  |
| 10.05 |  | ALL | Sept 22, 2009 |  |
| 10.06 | 34950 | ALL | Nov 10, 2009 |  |
| 10.09a | 35141 (beta release) | ALL | April 28 2010 | SS |
| 10.10 | 35159 | ALL | May 19, 2010 | SS |
| 10.20 | 35198 | ALL | Feb 25, 2011 | SS |
| 10.30 | 35616 | ALL | Aug 17, 2011 | SS |
| 10.31 | 35729 | ALL | April 11 2012 | SS |
| 10.40 | 35759 | ALL | Aug 3, 2012 | SS |
| 10.50 | 35867 | ALL | March 5 2013 | SS |
| 10.51 | 35886 | ALL | March 19, 2013 | SS |
| 10.52 | 35888 | ALL |  |  |
| 10.53 | 35895 | ALL | March 22, 2013 | SS |
| 10.60 | 35920 | ALL | May 21, 2013 | SS |
| 10.70G | XXXX | ALL | Sep 9, 2013 | TB |
| 10.70I | XXXX | ALL | Oct 10, 2013 | TB |
| 10.71 | 36031 | ALL | Mar 11, 2014 | TB |
| 10.72 | 36120 | ALL | Apr 16, 2014 | TB |

| Name | Version | Description | See Note |
| --- | --- | --- | --- |
| Blastware |  |  |  |
| BLASTWARE.EXE | 10.72 | Main Program |  |
| AUTOCALLHOME.EXE | 10.72 | Call Home using multiport program |  |
| INSTANTEL README.RTF | 10.72 | Readme release notes. |  |
| WBM3.CFG | 10.72 | OEM Configuration |  |
| BM3_OS.MOS | 4.32 | Firmware operating system |  |
| BM3_LOAD.MOS | 4.30 | RAM loader to program operating system. |  |
| BM3_OS6.MOS | 8.17 | Firmware OS for units with s/n >= 6000 |  |
| BM3LOAD6.MOS | 7.1 | RAM loader for units with s/n >= 6000 |  |
| MB_OS.MOS | 1.1 | Firmware OS for Minimate Blaster |  |
| SeriesIV_G4.BIN | 10.72 | Series IV application for G4 MSystems |  |
| SeriesIV_H3.BIN | 10.72 | Series IV applications for H3 MSystems |  |
| BlastwareHelp.pdf | 10.72 | Help file |  |
| BM3COMM32.DLL | 10.72 | Serial communications library |  |
| WCT32DR3.DLL | 10.72 | Graphic DLL (From QC but modified in house) |  |
| MICROMATE.BIN | 10.72 | Micromate Application Firmware |  |
| Micromate.inf | 10.72 | Micromate com port driver for USB |  |
| Usbser.sys | 6.1.7601.17514 | Microsoft system USB driver |  |
| Blastware\BWMail |  |  |  |
| BWMAIL.EXE | 10.72 | Email program |  |
| Tomail.dat | 8.0 | Queue of data to send is initialized by installation. |  |
| Blastware\Drivers |  |  |  |
| Blastware\Drivers\USBSerialConverter |  | USB to Serial driver for all Win OSs |  |
| Blastware\Event |  | Blank directory to contain user events |  |
| Blastware\Sample |  | Sample events directory |  |
| C00075EJB.230 | 7.1 | Sample event |  |
| Drift.bwp | 7.1 | Sample event |  |
| G73084ED.4X0 | 7.1 | Sample event |  |
| I0358WST.M10 | 7.1 | Sample event |  |
| Blastware\System\Comply |  |  |  |
| AS2187.CPL | 3.41 |  |  |
| BS7385.CPL | 1.51 | Compliance Definition |  |
| BSCRV128.CPL | 1.0 |  |  |
| BSCRV16.CPL | 1.51 |  |  |
| BSCRV20.CPL | 1.0 |  |  |
| BSCRV32.CPL | 1.0 |  |  |
| BSCRV60.CPL | 1.0 |  |  |
| BSCRV8.CPL | 1.51 |  |  |
| BSCRV90.CPL | 1.0 |  |  |
| CMRISTD.CPL | 3.0 |  |  |
| DGMSA.CPL | 3.51b |  |  |
| DGMSB.CPL | 3.51b |  |  |
| DIN4150.CPL | 1.0 |  |  |
| FRANCE.CPL | 1.0 |  |  |
| FRANCE2.CPL | 1.0 |  |  |
| FRANCE3.CPL | 3.3 |  |  |
| FRANCE4.CPL | 3.1 |  |  |
| GENERIC.CPL | 3.3 |  |  |
| INDIAN.CPL | 3.41 |  |  |
| NBR9653-05.cpl | 10.0 |  |  |
| NEWZEAL.CPL | 1.0 |  |  |
| NZSISO.CPL | 4.0 |  |  |
| SPAIN.CPL | 1.0 |  |  |
| SWEDISH.CPL | 3.2 |  |  |
| SWEDISH2.CPL | 3.2 |  |  |
| SWISSM.CPL | 3.0 |  |  |
| SWISSP.CPL | 3.0 |  |  |
| SWISST.CPL | 3.0 |  |  |
| USBMOSMR.CPL | 1.0 |  |  |
| QUEENSLAND.CPL | 7.0 |  |  |
| Czech Standard.cpl | 8.0 |  |  |
| Indonesian SNI 7571 2010 | 10.09a |  |  |
| Turkey – Construction.cpl | 10.20 |  |  |
| Turkey – Mining and Quarry.cpl | 10.20 |  |  |
| Toronto 154-2008.cpl | 10.40 |  |  |
| Blastware\System\Dbase |  |  |  |
| 45GEOMCT.ADV | 7.0 | Advanced configuration 4.5 Hz. Geophone. |  |
| BW6841WB.FWT | 8.0 | Frequency weight files |  |
| BW6841WD.FWT | 8.0 |  |  |
| BW6841WE.FWT | 8.0 |  |  |
| BW6841WF.FWT | 8.0 |  |  |
| BW6841WG.FWT | 8.0 |  |  |
| ISO2631WM.FWT | 8.0 |  |  |
| ISO263Wm-2003.FWT | 10.4 |  |  |
| BS6472Wg-2008.FWT | 10.4 |  |  |
| BS6472Wd-2008.FWT | 10.4 |  |  |
| BS6472Wb-2008.FWT | 10.4 |  |  |
| Def8chan.ADS | 8.0 |  |  |
| DEFAULT.EVM | 3.4 | Default event manager setup |  |
| Default.flx | 8.0 | Default setup files |  |
| Default.tas | 8.0 |  |  |
| Default.tes | 8.0 |  |  |
| Default.tvm | 8.0 |  |  |
| H15min.ads | 8.0 |  |  |
| H5sec.ads | 8.0 |  |  |
| DESKJET.DRV | 1.0 | Firmware diver for Deskjet printer |  |
| DICONIX.DRV | 1.0 | Firmware diver for Diconix printer |  |
| EPSON.DRV | 1.0 | Firmware diver for Epson printer |  |
| FACTORY.DBS | 8.0 | Setup (Database) factory defaults |  |
| LOGOMAP.DBS | 1.0 | Firmware OEM logo on printout. |  |
| LOGOBMAPVT.DBS | 1.0 | Firmware OEM logo on printout. |  |
| STDIMP.ADV | 7.0 | Default advanced configuration (imperial units) |  |
| STDMET.ADV | 7.0 | Default advanced configuration (metric units) |  |
| Blastware\system\fonts |  |  |  |
| SFONT.FNT | 1.0 | Firmware font file for printing (small font) |  |
| MFONT.FNT | 1.0 | Firmware font file for printing (medium font) |  |
| Blastware\System\Lang |  |  |  |
| ENGLISH.LNG | 8.12 | Language file |  |
| englishIV.lng | 10.50 | Language file |  |
| Germaniv,lng | 10.10 |  |  |
| DUTCH.LNG | 8.12 | Language file |  |
| FRENCH.LNG | 8.12 | Language file |  |
| ITALIAN.LNG | 8.12 | Language file |  |
| PORTUGESE(BRAZIL).LNG | 8.12 | Language File |  |
| SPANISH.LNG | 10.0 |  |  |
| Eurodate.upg | 4.02 | Uses European date format. |  |
| Blastware\System\Modem |  |  |  |
| GEN_AT.MDM | 3.5 | Generic modem driver |  |
| HAYES.MDM | 3.5 | Hayes modem driver |  |
| USR5630.MDM | 4.32 | USR modem driver for the 5630 |  |
| WAVECOM.MDM | 10.0 | Wavecom modem driver |  |
| OEMLOGO.BMP | 7.0 | OEM bitmap for print logo. |  |
| NLSPLASH.BMP | 7.0 | OEM bitmap for splash screen. |  |
| Blastware\System\Series2 |  |  |  |
| MAINLOAD.DWN |  | Series 2 Operating System |  |
| AMOS2.DWN | 5.52 |  |  |
| AMOSEV5.DWN |  |  |  |
| amosmini.dwn | 2.61 | Minimate V2.61 |  |
| AMOSMS5.DWN |  |  |  |
| bmos2.dwn | 5.52 | Blastmate V5.52 |  |
| BMOSEV5.DWN |  |  |  |
| BMOSMINI.DWN | 2.61 |  |  |
| BMOSMS5.DWN |  |  |  |
| cmos2.DWN | 5.52 |  |  |
| CMOSEV5.DWN |  |  |  |
| CMOSMINI.DWN | 2.61 |  |  |
| CMOSMS5.DWN |  |  |  |
| DMOS2.DWN | 5.52 |  |  |
| DMOSEV5.DWN |  |  |  |
| DMOSMINI.DWN |  |  |  |
| DMOSMS5.DWN |  |  |  |
| BSCRV128.FRQ | 3.74 |  |  |
| BSRV16.FRQ | 3.74 |  |  |
| BSCRV20.FRQ | 3.74 |  |  |
| BSCRV32.FRQ | 3.74 |  |  |
| BSCRV60.FRQ | 3.74 |  |  |
| BSCRV8.FRQ | 3.74 |  |  |
| BSCRV90.FRQ | 3.74 |  |  |
| BSCVr20.FRQ | 3.74 |  |  |
| DIN4150.FRQ | 3.74 |  |  |
| FRANCE.FRQ | 3.74 |  |  |
| FRANCE2.FRQ | 3.74 |  |  |
| NEWZEAL.FRQ | 3.74 |  |  |
| SPAIN.FRQ | 3.74 |  |  |
| USBMOSMR.FRQ | 3.74 |  |  |
| LOGOBMAP.DBS | 3.74 |  |  |
| Hydrophone.nsa |  | Advanced Series IV setup files |  |
| High Pressure Mic.nsa | 10.50 |  |  |
| High Frequency Geo.nsa | 10.50 |  |  |
| 500g Acceletometer.nsa | 10.50 |  |  |
| 50g Acceletometer.nsa | 10.50 |  |  |
| 5g Acceletometer.nsa | 10.50 |  |  |

NorthStar Verification Checklist

# Unit Interface

Unit Specific:

|  | Oem Family. | 10.50 Mar 5/13 |  |
| --- | --- | --- | --- |
|  | Oem Model. | 10.50 Mar 5/13 |  |
|  | Calibration name and date | 10.50 Mar 6/13 |  |
|  | Serial number (correct OS loaded?) | 10.50 Mar 5/13 |  |
|  | Battery level. Check recharge and Overcharging states. Check for min. update of once per hour during operation???? |  |  |
|  | Time displayed properly | 10.50 Mar 5/13 |  |
|  | Date | 10.50 Mar 5/13 |  |
|  | Keypad | 10.50 Mar 5/13 |  |

Change Setup File

|  | Select Setup File | 10.50 Mar 5/13 |  |
| --- | --- | --- | --- |
|  | Delete Setup File | 10.50 Mar 5/13 |  |

Change Operator

View/Edit Setup File – Compliance Setup File

|  | Record mode – Waveform, manual, histogram, combo, VDV | 10.50 Mar 5/13 |  |
| --- | --- | --- | --- |
|  | Vibration Sensor – ISEE geo, DIN 315 geo, DIN 80 geo, No Sensor | 10.50 Mar 5/13 |  |
|  | Pressure - ISEE geo, DIN 315 geo, DIN 80 geo, Linear mic, Sound Level Mic, No Sensor | 10.50 Mar 5/13 |  |
|  | Sample Rate – 512, 1024, 2048, 4096 | 10.50 Mar 5/13 |  |
|  | Sound Mic Setup SLM not selected: N/A – N/A SLM selected: A weighting – Fast A weighting – Slow C weighting – Fast C weighting – Slow | 10.50 Mar 5/13 |  |
|  | Sound Mic Setup screen Mic Weighting: A weighting, C weighting Mic Response: Fast, Slow | 10.50 Mar 5/13 |  |
|  | Trigger Level: Vib 2.54 OP 50.0 Vib 2.54 OP 50.0 Vib 2.54 SL 50.0 Vib 2.54 SL 50.0 | 10.51 Mar 7/13 |  |
|  | Trigger Level screen: Vibration 1 Trigger: On, Off Vibration 1 Trigger Level: should limit to appropriate range. Pressure Trigger: On, Off Pressure Trigger Level: should limit to appropriate range, should show correct units. | 10.50 Mar 6/13 | Should not have pressure for slm? |
|  | Fixed record time @ 1 and max seconds for a given setup | 10.50 Mar 6/13 |  |

View/Edit Setup File – Advanced Setup File

|  | Record mode – Waveform, manual, histogram | 10.52ad Mar 18/13 |  |
| --- | --- | --- | --- |
|  | Channels 1-3 & 4-6 - Channel 1 name, Off, Off Off, Off, Off - Channel 1 name, Channel 2 name, Off Off, Off, Off - Channel 1 name, Channel 2 name, Channel 3 name Off, Off, Off - Channel 1 name, Channel 2 name, Channel 3 name Channel 4 name, Off, Off - Channel 1 name, Channel 2 name, Channel 3 name Channel 4 name, Channel 5 name, Off - Channel 1 name, Channel 2 name, Channel 3 name Channel 4 name, Channel 5 name, Channel 6 name | 10.50 Mar 6/13 |  |
|  | Sample Rate – 512, 1024, 2048, 4096, 8192, 16384, 32768, 65536 | 10.52ad Mar 18/13 |  |
|  | Trigger Level: View/Edit | 10.50 Mar 6/13 |  |
|  | Trigger Level screen: Trigger chan 1: On, Off Trigger Level chan 1: 10.0 pa … Trigger chan 6: On, Off Trigger Level chan 6: 10.0 pa | 10.51 Mar 13/13 |  |
|  | Trigger Level Sound screen: If Blastware’s report options were setup for metric units when setup file was transferred to unit: Range: Sensitive (+/- 25 pa), Normal (+/- 100 pa) Trigger Level: 10.0 pa If Blastware’s report options were setup for imperial units when setup file was transferred to unit: Range: Sensitive (+/- 0.003625 psi), Normal (+/- 0.0145 psi) Trigger Level: 10.0 pa | 10.52ad Mar 18/13 |  |
|  | Trigger Level for SLM edit screen: Normal range: Results: Limits: 0.000061 to 0.0145 psi. 0.0000650 – 0.0145 psi (unit) 0.00006496 – 0.0145 psi (BW) Limits: 0.42105 to 100 pa 0.448 – 100 pa (unit) 0.448 – 100 pa (BW) Sensitive range: Limits: 0.00000193 to 0.000363 psi. 0.0000021 – 0.00363 psi (unit) 0.000002064 – 0.003625 (BW) Limits: 0.0133 to 25 pa 0.0142 – 25 pa (unit) 0.01424 – 25 pa (BW) | 10.51 Mar 13/13 |  |
|  | Fixed record time @ 1 and max seconds for a given setup |  |  |

View/Edit Preferences

|  | Daily Self Check: Disabled |  |  |
| --- | --- | --- | --- |
|  | Daily Self Check Time |  |  |
|  | Sensorcheck Disabled |  |  |
|  | Sensorcheck Before Monitoring |  |  |
|  | Sensorcheck After Event |  |  |
|  | Imperial/Metric | 10.51 Mar 13/13 |  |
|  | Daily Start Time (all modes) |  |  |
|  | Daily Stop Time (all modes) |  |  |
|  | One shot Start Time (all modes) |  |  |
|  | One shot Stop Time (all modes) |  |  |
|  | No Timer |  |  |
|  | O/P (microphone) units (dB or linear) |  |  |
|  | Beep On Trigger |  |  |
|  | Beep On Keypress |  |  |
|  | Backlight (on or off) |  |  |
|  | Backlight Timeout |  |  |
|  | LCD Auto Off |  |  |
|  | Current Time – edit |  |  |
|  | Direct Serial Connection at 9600, 38400, 115200 |  |  |
|  | Reset to Factory Defaults |  |  |

Maintenance Menu

|  | About | 10.52af, Mar 19/13 |  |
| --- | --- | --- | --- |
|  | View Channel | 10.52ad, Mar 18/13 |  |
|  | Disconnect Battery | 10.52ad, Mar 19/13 |  |
|  | Sound Level Microphone – Field Calibration | 10.50 Mar 5/13 |  |

View Data Files

|  | View Events in unit: | 10.52ad, Mar 18/13 |  |
| --- | --- | --- | --- |
|  | Does not scroll beyond existing events. | 10.51 Mar 13/13 |  |
|  | Delete All Events in unit | 10.51 Mar 13/13 |  |
|  | Delete Selected Event | 10.51 Mar 13/13 |  |
|  | Delete Marked Events | 10.51 Mar 13/13 |  |
|  | Delete All Events using Blastware | 10.51 Mar 13/13 |  |
|  | Mark/unmark events | 10.51 Mar 13/13 |  |

Others

|  | Analog board calibration (initiated by Blastware) should function as expected. | 10.52af, Mar 19/13 |  |
| --- | --- | --- | --- |

# Monitoring and Recording

Monitoring  and Recording

| Feature/Test | Desired Result | Actual Result | Date | Initial |  |
| --- | --- | --- | --- | --- | --- |
| Record events with all channel combinations. |  | 1,3,4,6 1 | Mar 5/13 Mar 18/13 | PS |  |
| Trigger Source | Unit should correctly detect the trigger channel. | It correctly detects: Vert, Long, SLM, linear mic | Mar 6/13 Mar 13/13 | PS |  |
| Sample Rates |  |  |  |  |  |
| Record events with each sample rate. |  | Waveform: 512, 1024, 2048, 4096, 65536 Histogram: 512, 1024, 2048, 4096, 65536 Combo: 512, 1024, 2048, 4096 Waveform: 1024, 16384 | Mar 13/13 Mar 18/13 | PS |  |
| Record length |  |  |  |  |  |
| Record events with both Fixed Time and Auto Record stop modes. | Event lengths should match the set length. | Test both record stop modes. | Mar 13/13 | PS |  |
| Record events with various record lengths. | 1, 2, 5, 10, 20, 30 second events should contain that much data. | 1, 2, 10, 30 1, 5, 90 1, 2, 5, 7 | Mar 5/13, Mar 13/13 Mar 18/13 | PS |  |
| Record events with various auto stop windows. | Event lengths should match the set length. | 1, 3 & 9 seconds | Mar 13/13 |  |  |
| Waveform event cut short due to full memory or cancel key. | Reported record length should match the amount of actual post-trigger data recorded. | It does. | Mar 12/13 | PS |  |
| Record histograms with various interval sizes. |  | Recorded using 2, 5, 30 second, 1 and 60 minute intervals. | Mar 13/13 | PS |  |
| Histograms should contain the correct number of intervals | A long histogram should report the correct number of intervals. | A ~14 hour histogram which wrapped for one day to another contained the correct number of intervals. | Mar 13/13 | PS |  |
| Unit should still correctly detect triggers when recording a long histogram. |  | Unit correctly detected a trigger after ~14 hours into a histogram. | Mar 6/13 | PS |  |
| Waveform event triggered immediately upon entering monitor mode. | The pretrigger should be limited. |  | Mar 13/13 | PS |  |
| Waveform event triggered immediately following a previous waveform event. | The pretrigger should be limited so as to not include data from the previous event. | It is and does not. | Mar 13/13 | PS |  |
| The correct number of samples are included in a waveform events. |  |  | Mar 13/13 | PS |  |
| There should be no dead time between events when signal continuously exceeds trigger level. | Setup input signal and trigger level such that signal is always above trigger level. Used a special version of the f/w which dumped the event start and trigger sample indices. | There is no dead time between events. | Mar 12/13 | PS |  |
| Events should not overlap. | Setup input signal and trigger level such that signal is always above trigger level. Used a special version of the f/w which dumped the event start and trigger sample indices. | There is no overlap in events. | Mar 12/13 | PS |  |
| Record 20+ waveform events during a single histogram | Unit should continue to trigger waveform events. | Recorded 80 waveforms during a single histogram. | Mar 6/13 | PS |  |
| Histograms recorded at 65536 sps on 6 channels | should not kick out after a few intervals |  | Mar 12/13 | PS |  |
| Calculations |  |  |  |  |  |
| Peak | The correct peak is reported when said peak is during pretrigger. |  | Mar 5/13 | PS |  |
|  | The correct peak is reported when said peak is is during post-trigger. |  | Mar 12/13 | PS |  |
| Time of peak | The correct time of peak is reported when said peak is during pretrigger. |  | Mar 5/13 | PS |  |
|  | The correct time of peak is reported when said peak is is during post-trigger. |  | Mar 12/13 | PS |  |
| PVS | The correct PVS is reported when it during pretrigger. |  | Mar 5/13 | PS |  |
|  | The correct PVS is reported when it occurs during post-trigger. |  | Mar 13/13 | PS |  |
| OTHER CALCS? |  |  |  |  |  |

## Monitor Scheduler Off

### Monitor mode Initiated from Keypad with Scheduler off

Test in all record modes.

For each of the following setups:

- Press start monitor key.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only | Combo, H3, 10.45bq, Feb 27, 2013 |  |  |
| Geo + mic | Manual, G4, 10.45bp, Feb 27, 2013 Historam, G4, 10.45bp, Feb 27, 2013 Combo, H3, 10.45bq, Feb 27, 2013 | Waveform, G4, 10.50, Mar 6, 2013 | Waveform, G4, 10.50, Mar 5, 2013 |
| Mic only | Waveform, H3, 10.45bq, Feb 27, 2013 Waveform, G4, 10.45bq, Feb 27, 2013 |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

### Monitor mode Initiated from Blastware with Scheduler off

Test in all record modes.

For each of the following setups:

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only |  |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

### Advanced mode Initiated from Keypad with Scheduler off

For each of the following setups:

- Press start monitor key.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Test | Results | Initials / Date |
| --- | --- | --- |
| Waveform, 6 channels included an SLM in normal mode at 65536 sps | Recorded a 10 second event as it should. No errors. | PS, Mar 5, 2013 |
| Histogram, 6 channels included an SLM in normal mode at 65536 sps | Recorded for 130 intervals with out stopping. No errors. | PS, Mar 5, 2013 |

### Advanced mode Initiated from Blastware with Scheduler off

For each of the following setups:

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Test | Results | Initials / Date |
| --- | --- | --- |
| Waveform, 4 channels included an SLM in sensitive mode at 65536 sps | Recorded a 30 second event as it should. No errors. | PS, Mar 5, 2013 |

## Monitor Scheduler On

### Monitor mode Initiated from Keypad with Scheduler On

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode.

- Press start monitor key.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only | Histogram, H3, 10.45bq, Feb 28, 2013 |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters scheduled monitor mode but will enter active monitor mode in ~2 minutes.  i.e. schedule puts unit into monitor mode at 10:00 and unit time set for 9:57.

- Press start monitor key.

- Wait until unit enters active monitor mode.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only |  |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode but will perform a self check in ~2 minutes.  i.e. schedule performs self check at 10:00 and unit time set for 9:57.

- Press start monitor key.

- Wait until unit enters active monitor mode after the self check.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only |  |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

### Monitor mode Initiated from Blastware with Scheduler On

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode.

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only |  |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters scheduled monitor mode but will enter active monitor mode in ~2 minutes.  i.e. schedule puts unit into monitor mode at 10:00 and unit time set for 9:57.

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Wait until unit enters active monitor mode.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only |  |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode but will perform a self check in ~2 minutes.  i.e. schedule performs self check at 10:00 and unit time set for 9:57.

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Wait until unit enters active monitor mode after the self check.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Geo only |  |  |  |
| Geo + mic |  |  |  |
| Mic only |  |  |  |
| Geo + geo (ISEE) |  |  |  |
| Geo + geo (DIN 315) |  |  |  |
| Geo + geo (DIN 80) |  |  |  |

### VDV mode Initiated from Keypad with Scheduler On

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode.

- Press start monitor key.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Tran only | 10.51 Mar 13/13 |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters scheduled monitor mode but will enter active monitor mode in ~2 minutes.  i.e. schedule puts unit into monitor mode at 10:00 and unit time set for 9:57.

- Press start monitor key.

- Wait until unit enters active monitor mode.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Vert only |  | 10.51 Mar 13/13 |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode but will perform a self check in ~2 minutes.  i.e. schedule performs self check at 10:00 and unit time set for 9:57.

- Press start monitor key.

- Wait until unit enters active monitor mode after the self check.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Vert and Long only |  |  | 10.51 Mar 13/13 |

### VDV mode Initiated from Blastware with Scheduler On

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode.

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Tran, Vert, Long |  |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters scheduled monitor mode but will enter active monitor mode in ~2 minutes.  i.e. schedule puts unit into monitor mode at 10:00 and unit time set for 9:57.

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Wait until unit enters active monitor mode.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Tran + Vert |  |  |  |

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode but will perform a self check in ~2 minutes.  i.e. schedule performs self check at 10:00 and unit time set for 9:57.

- Press “Start monitoring” button in monitor dialog box.

- Press “Accept Setup and Monitor”.

- Wait until unit enters active monitor mode after the self check.

- Trigger a single event.

- Press “Stop monitoring” button in monitor dialog box.

- Check service log for error messages.

| Sensorcheck \ Channels | None | Before Monitoring | After Each Event |
| --- | --- | --- | --- |
| Vert + Long only |  |  |  |

### Advanced Waveform mode Initiated from Keypad with Scheduler On

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode.

- Press start monitor key.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Test | Results | Initials / Date |
| --- | --- | --- |
| HF Geo + SLM | Event recorded as it should be. No errors reported. | PS, 10.50, Mar 6/13 |

For each of the following setups:

- Setup schedule and unit time so that unit enters scheduled monitor mode but will enter active monitor mode in ~2 minutes.  i.e. schedule puts unit into monitor mode at 10:00 and unit time set for 9:57.

- Press start monitor key.

- Wait until unit enters active monitor mode.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Test | Results | Initials / Date |
| --- | --- | --- |
| HF Geo + SLM | Event recorded as it should be. No errors reported. | PS, 10.50, Mar 6/13 |

For each of the following setups:

- Setup schedule and unit time so that unit enters active monitor mode but will perform a self check in ~2 minutes.  i.e. schedule performs self check at 10:00 and unit time set for 9:57.

- Press start monitor key.

- Wait until unit enters active monitor mode after the self check.

- Trigger a single event.

- Cancel monitoring.

- Check service log for error messages.

| Test | Results | Initials / Date |
| --- | --- | --- |
| HF Geo only |  |  |

# Sensorcheck

| Sensorcheck \ Channels | Keypad | Blast ware | Before Monitoring | After Each Event | Self Check |
| --- | --- | --- | --- | --- | --- |
| ISEE geo connected; ISEE geo enabled; | 10.51 Mar 12/13 | 10.51 Mar 12/13 |  |  |  |
| DIN 315 geo connected; ISEE geo + SLM enabled | 10.51 Mar 7/13 |  | n/a | n/a | n/a |
| Linear Mic connected; Linear mic enabled | 10.51 Mar 12/13 | 10.51 Mar 12/13 | 10.51 Mar 12/13 | 10.51 Mar 12/13 |  |
| Linear Mic connected; SLM enabled | 10.51 Mar 12/13 | 10.51 Mar 12/13 | n/a | n/a | n/a |
| SLM connected; SLM enabled | 10.51 Mar 13/13 | 10.51 Mar 13/13 | 10.51 Mar 13/13 |  | 10.51 Mar 7/13 |
| SLM connected; linear enabled | 10.51 Mar 13/13 | 10.51 Mar 13/13 | n/a | n/a | n/a |
| ISEE Geo + Linear Mic connected; ISEE Geo + Linear Mic enabled | 10.51 Mar 13/13 | 10.51 Mar 13/13 | 10.51 Mar 13/13 | 10.51 Mar 13/13 |  |
| ISEE Geo + Linear Mic connected; ISEE geo + SLM enabled | 10.51 Mar 13/13 |  |  |  |  |
| ISEE geo + SLM connected; ISEE geo + SLM enabled | 10.51 Mar 13/13 | 10.51 Mar 13/13 | 10.51 Mar 13/13 | 10.51 Mar 13/13 |  |
| Geo + geo (ISEE) connected; DIN 315 & SLM enabled | 10.50ao Mar 6/13 | 10.50ao Mar 6/13 | n/a | n/a | n/a |
| Geo + geo (ISEE) connected; Geo + geo (ISEE) enabled; | 10.50ao Mar 6/13 | 10.50ao Mar 6/13 | 10.50ao Mar 6/13 |  |  |
| DIN 315 geo + SLM connected; DIN 315 geo + SLM enabled | 10.51 Mar 7/13 | 10.51 Mar 7/13 | 10.51 Mar 7/13 | 10.51 Mar 7/13 |  |
| DIN 315 geo + SLM connected; DIN 315 geo + linear enabled | 10.51 Mar 7/13 | 10.51 Mar 7/13 | n/a | n/a | n/a |
| DIN 315 geo + SLM connected; DIN 315 geo enabled | 10.51 Mar 7/13 | 10.51 Mar 7/13 | n/a | n/a | n/a |
| DIN 315 Geo + ISEE geo connected; SLM enabed | 10.51 Mar 7/13 | 10.51 Mar 7/13 | n/a | n/a | n/a |
| Geo + geo (DIN 80) |  |  |  |  |  |

|  | Amplitude too low and too high must be correctly detected and reported for all channels. |  |
| --- | --- | --- |
|  | Overswing ratio too low and too high must be correctly detected and reported for all channels. |  |
|  | Frequency too low and too high must be correctly detected and reported for all channels. |  |
|  | Geo channel: error reported if 1st peak > ? counts (WAS -30 COUNTS ON SERIES III) |  |
|  | Geo channel: error reported if 2nd peak < ? counts (WAS 7 COUNTS ON SERIES III) |  |
|  | Geo channel: error reported 1st peak < 2nd peak or if 2nd peak < 3rd peak. |  |
|  | Linear Mic channel: error reported if less than 3 positive deltas > ? counts (WAS 300 COUNTS ON SERIES III) |  |
|  | Sensorcheck should pass when the sensorcheck waveforms are correct and should fail when the sensorcheck waveforms are incorrect. |  |
|  | Sensorcheck before monitoring should not cause a false trigger with the minimum trigger level. |  |
|  | SLM, passes if 400 mV < amplitude < 1000 mV | 10.50 Mar 5/13 |
|  | Should fail if amplitude > 1000 mV | 10.50 Mar 5/13 |
|  | Sensorcheck after an event should not cause a false trigger with the minimum trigger level. |  |
|  | Sensorcheck initiated by the Sensor Check button should pass when the sensorcheck waveforms are correct and should fail when the sensorcheck waveforms are incorrect. | 10.50 Mar 5/13 |
| - W | When setup to perform a sensorcheck before monitoring those sensorcheck results should be included in all events recorded during the monitoring session. |  |

# SLM

| Test | Results | Initials / Date |
| --- | --- | --- |
| Mic should be setup for the time constant selected by the user. Use B&K 4226 in Time Weighting S function. When mic is setup for fast time constant the pulse should be squarish with rounded corners as it approaches the end of the change in level. When mic is setup for slow time constant the pulse should be look like a shark fin with no flat top because it had not finished rising before the input started back down. | The pulses captured with both time constants are as expected. | PS, 10.51, Mar 13/13 |
| Mic should be put in advanced normal range when setup to do so. | It is. | PS, 10.52a9, Mar 18/13 |
| Mic should be put in advanced sensitive range when setup to do so. | It is. | PS, 10.52a9, Mar 18/13 |
| Extended notes should not contain debugging info about range, smoothing, etc. | It does not. | PS, 10.50, Mar 6/13 |

Range changes should be smoothed over.

Fast Time constant

Test in waveform and combo modes.

| Sample Rate Range Change | 512 | 1024 | 2048 | 4096 |
| --- | --- | --- | --- | --- |
| Low to medium | Waveform, 10.45cf, Mar 4, 2013 Combo, 10.45cf, Mar 4, 2013 | Waveform, 10.45ci, Mar 5, 2013 | Waveform, 10.45ci, Mar 5, 2013 Combo, 10.50, Mar 5, 2013 | Combo, 10.45cd, Mar 4, 2013 |
| Medium to high | Waveform, 10.45cf, Mar 4, 2013 Combo, 10.45cf, Mar 4, 2013 | Waveform, 10.45ci, Mar 5, 2013 | Combo, 10.50, Mar 5, 2013 | Combo, 10.45cd, Mar 4, 2013 |
| High to medium | Waveform, 10.45cf, Mar 4, 2013 Combo, 10.45cf, Mar 4, 2013 | Waveform, 10.45ci, Mar 5, 2013 | Combo, 10.50, Mar 5, 2013 | Combo, 10.45cd, Mar 4, 2013 |
| Medium to low | Waveform, 10.45cf, Mar 4, 2013 Combo, 10.45cf, Mar 4, 2013 | Waveform, 10.45ci, Mar 5, 2013 | Waveform, 10.45ci, Mar 5, 2013 Combo, 10.50, Mar 5, 2013 | Combo, 10.45cd, Mar 4, 2013 |

Slow Time constant

Test in waveform and combo modes.

| Sample Rate Range Change | 512 | 1024 | 2048 | 4096 |
| --- | --- | --- | --- | --- |
| Low to medium | Waveform, 10.45ci, Mar 5, 2013 | Waveform, 10.50, Mar 5, 2013 Combo, 10.50, Mar 5, 2013 | Waveform, 10.50, Mar 5, 2013 | Waveform, 10.45ci, Mar 5, 2013 |
| Medium to high | Waveform, 10.45ci, Mar 5, 2013 | Waveform, 10.50, Mar 5, 2013 Combo, 10.50, Mar 5, 2013 | Waveform, 10.50, Mar 5, 2013 | Waveform, 10.45ci, Mar 5, 2013 |
| High to medium | Waveform, 10.45ci, Mar 5, 2013 | Waveform, 10.50, Mar 5, 2013 Combo, 10.50, Mar 5, 2013 | Waveform, 10.50, Mar 5, 2013 | Waveform, 10.45ci, Mar 5, 2013 |
| Medium to low | Waveform, 10.45ci, Mar 5, 2013 |  | Waveform, 10.50, Mar 5, 2013 | Waveform, 10.45ci, Mar 5, 2013 |

| Test | Results | Initials / Date |
| --- | --- | --- |

## In-field Calibration

Requires a calibration reference source.

|  | If calibration reference is connected then calibration should pass. | 10.50 Mar 5/13 |
| --- | --- | --- |
|  | If calibration reference is not connected then calibration should fail and report that calibration reference is not detected. | 10.50 Mar 5/13 |
|  | If signal is too large (i.e. calibration reference set for 114 dB) then calibration should fail and report that calibration reference is too large. | 10.45bd, Feb 25/13 |

BW Series IV

| Compliance Setup Add SLM Weighting Response Edit LN If SLM on the channel name is Lmax. Editing the alarm levels Check LN values for range | SLM, weighting, response and Exceedance levels can be edited and saved/read to/from the unit. Check boxes function in an expected manor. Channel 4 names: “Lmax” for SLM, “MicL” for linear mic and “Tran2”. If a user edited name is found the names are not overwritten. Huge numbers in the LN fields resulted in bad range checking. Fixed. |
| --- | --- |
| Event Report Add L(N1) and L(N2) to the Report options. The N1 or N2 should be replaced the actual level being reported. Plotting Leq, L10, L90 so that labels do not fall on top of each other. Overload indicator for SLM at 140dB. | LN1 and LN2 were reversed. Fixed. If LN1 = 10 the L10 level is displayed in the unit and on the event reports. The labels are offset in the event report so do not overwrite each other. In Compliance mode the SLM sensor result is displayed as “***” if it is over 140 dB. Compliance WF - OK Histogram - OK Advanced WF - OK Histogram - OK |
| Event manager Exceedance and Sound for L(N1) column and L(N2) column Exceedance is the Level and Sound is the calculated level in dB | Added OK Deleted OK reports correct values OK |
| Sensorcheck | Correct number of channels1-6 displayed on Pro6 Naming of SLM channel-OK |
| Monitor mode from Blastware | SLM labeled - OK |
| Convert to ASCII Output Sound monitoring string as in event report Frequency of SLM events is included in Advanced histogram events. | Compliance WF - OK Histogram - OK Advanced - OK WF - OK Histogram - OK |
| Advanced setup screen Units and sensitivity grayed out when SLM Normal/Sensitive check boxes. Prevent active channel from having sensitivity of zero or blank. Check Ranges when switching between sensitive and normal. | OK |
| Sound Monitoring - Calculations | Available for advanced WF events |
| Sound Monitoring – Octave Data Conversion | OK |
| Only redraw the advanced plot once after processing. In general ensure that eliminating the multiple redraw has no unexpected effects. | OK |
| On Unit Unable to Monitor \n Memory critically low message. | OK |
| Editing alarm level of SLM in Blastware | Range checked - OK |
| Copyright changed to 2013 in Blastware | BW, BWCAll BWMail - OK |
| Startup Screen Update the Start-up screen to show what has been completed | OK |
| Install disk – copyright 2013 | OK |
| ACH event SLM reports match Blastware event reports. | Compliance WF - OK Histogram - OK Advanced WF - OK Histogram - OK |
| ACH convert to ASCII and XML match the Blastware ASCII file. | Add Peak limiting to 140dB. - OK XML key in “Sound (dB)” changed to “Sound” |
| Startup Screen Copy manuals to selected directory. Initial default directory. Create if needed. | OK |
| Scheduler controlled monitoring and ACH. Check that it does not restart monitoring | OK |
| On Unit Call Home is scheduler mode | Timed ACH - OK Timed ACH with monitor while calling – OK Found and fixed the problem that caused the unit not to go back to monitor mode. |
| On Unit Call Home no scheduler | Checked geo and Mic trigger. |
| Pre-trigger in advanced mode | Was set to 0 in Blastware .. Fixed and tested |
| SLM calibration set to FieldCalibrat displayes as “Field Calibration”. | OK |
| Auto Call Home on unit without being attached to an ACH program so that every call fails. | Restart is not perfect. A Start Monitor in the scheduler every half hour. Restarts did not always occur in half hour of the failure. Sometimes starting monitoring was initiated by the next Start Monitor and not be the Call Home retry logic. |
| Check that the documentation distributed in the release is correct. | Bruce checked. |

Series III

| ACH 10.51 | SIII call home with Radio Ring. Waveform: Compared the event reports and found that the peak time of the mic was not displayed in Blastware. Fixed in 10.52 Histogram: OK |
| --- | --- |
| Handle SLM events | OK |
| Series III Selection Check Box | OK |
| Correct menus displayed for SIII |  |
| Date – Time setup Correct dialog, function. | OK |
| Monitor dialog. Correct options and function. | OK |
| Monitor Log | OK |
| Reload Operating system | OK |
| ACH and BWMail start | OK |
| Status reported correctly | OK |
| Compliance setup | OK |
| Compliance setup More | OK |
| Advanced setup | OK |
| Recognizes a SIII as opposed to a Blaster. | OK |
| Sensor Check (Unit Channel test) | Perform Test, Plot Waveform, Fetch Results OK |
| Reload Unit | OK |
| View Events | H, W, AW, AH and SLM events OK |
| Quick Report | H, W, AW, AH and SLM events OK |
| Analysis view | H, W, AW, AH and SLM events OK |
| Copy events to PC | OK |
| Delete events on the unit | OK |
| Quick Report matches event data displayed on the unit. | H, W, AW, AH OK |

‘A’ Weight in Series III tested by Soren and Paul for 10.53